|  | Assist Control A/C | Pressure Control PCV | Pressure regulated volume control PRVC | Synchronized Intermittent Mandatory Ventilation SIMV | Pressure Support Ventilation PSV | Volume Support V/S |
| --- | --- | --- | --- | --- | --- | --- |
| MODE | Control | Control | Control | Support | Support | Support |
| PRESET | Tidal Volume Resp Rate | Upper airway pressure Resp Rate | Tidal volume Resp Rate | Tidal volume Resp Rate | Pressure level and PEEP | Tidal Volume |
| VARIABLE | Pressure will vary according to lung compliance | Tidal volume based on pressure setting | Tidal volume may or may not remain constant based on lung compliance | Allows pt to take a spontaneous breath Supports spontaneous respirations with pressure | Strictly patient dependent mode | Strictly patient dependent mode |
| OTHER | Complete control No spontaneous breathing Not very safe Can be traumatic to airways | Patient’s with COPD require pressure | Lung protective mode Vent will decrease pressure by 5 3 times if it senses a problem | Used for quick weaning (<6 hours) especially CVRU patients | Pt must be breathing on their own Consider meds that can alter the respiratory drive | Patient must be breathing on their own. Each patient-triggered breath will receive set tidal volume |
| WEANING MODE | NO | NO | NO | YES | YES | YES |